ESOPHAGEAL MOTILITY DISORDERS ON HIGH RESOLUTION MANOMETRY:

CHICAGO CLASSIFICATION VERSION 4.0©

INTRODUCTION

The Chicago Classification categorizes esophageal motility disorders via an algorithmic scheme using metrics from esophageal high-resolution manometry (HRM). The first full version of the Chicago Classification was published in 2009, with two updates, including the current version 3.0 published in 2015.1-3 Over the past 5 years, both clinical applications and research investigations involving HRM have expanded, with introduction of novel HRM metrics. Therefore, an International HRM Working Group of 52 members worked over two years to develop the Chicago Classification version 4.0 (CCv4.0). This overview document will summarize the CCv4.0 initiative and present the updated modern-day Chicago Classification of esophageal motility disorders.

METHODS

The CCv4.0 Working Group is comprised of 52 members selected by six international motility societies and represents 20 countries. Characteristics of the working group are detailed in Supplemental Table 1. The CCv4.0 initiative was a two-year process (November 2018 to October 2020) which included three international meetings (May 2019, San Diego, CA; October 2019, Barcelona, Spain; August 2020, web-conference), multiple sub-group meetings, and seven surveys.

An initial survey conducted in January 2019 identified priority areas for update and modification from the previous 3.0 version (Supplemental Table 1). Accordingly, members were assigned to seven sub-groups: Standard HRM protocol, Achalasia, Esophagogastric junction (EGJ) outflow obstruction (EGJOO), Distal esophageal spasm (DES), Hypercontractile esophagus, Ineffective esophageal motility (IEM), and EGJ metrics. Each sub-group was led by two co-chairs and included a non-voting member who independently reviewed supportive literature and assessed level of evidence. Co-chairs and sub-group members were tasked with developing statements to support conclusive diagnosis of the motility disorder assigned to their sub-group, and to describe inconclusive scenarios for motility disorders which would require additional testing. These statements were based on literature review and expert consensus.

In addition to expert consensus, a priority for CCv4.0 was to utilize formal validated methodologies to determine both appropriateness of statements, and level of supportive evidence for each statement. The RAND Appropriateness Method (RAM) was utilized, with two rounds of independent electronic voting to determine appropriateness of each statement per RAM using University of California San Diego Research Electronic Database Capture (REDCap). Statements were considered appropriate when meeting >80% agreement, and are included in the final CCv4.0.4, 5 Statements with >85% agreement were considered strong recommendations, while those with 80 to 85% agreement were considered conditional recommendations. Statements nearly meeting criteria and/or those generating controversy were discussed at working group meetings. Additionally, statements that met criteria for inclusion in the final CCv4.0 underwent further independent evaluation to assess the level of supportive evidence, using the Grading of Recommendations Assessment, Development, and Evaluation (GRADE) process, when possible. Two experts external to the working sub-groups were assigned to independently evaluate the supportive literature provided by the sub-groups: one grader was an expert in the field external to the working sub-group and the second grader was a formally trained GRADE methodologist. Some statements were not amenable to the GRADE process, either because of the structure of the statement or lack of available evidence.

This document summarizes the final recommendations of the CCv4.0 working group. Separate technical reviews specific to each working group will summarize the statement development process, survey results, and supporting literature. These will be published separately in future issues of Neurogastroenterology and Motility over the coming months.

STANDARD HRM PROTOCOL & THRESHOLDS

Standard HRM Protocol

An important priority of the Chicago Classification was a standardized manometry protocol for motility laboratories around the world, to facilitate procedural consistency, improve diagnostic reliability, and promote collaborative research. The final standard esophageal HRM protocol is summarized below, in Figure 1, and depicted in Figure 2. Details regarding positions, maneuvers and bolus consistency are described in the subsequent technical review on HRM protocol.

The study begins in the supine position (Figure 2A). Following catheter placement, a minimum of 60 seconds of quiet rest allows for an adaptation period, following which catheter position is confirmed using a minimum of three deep inspirations. Next, a baseline period of at least 30 seconds is captured to enable identification of anatomic landmarks including the upper esophageal sphincter (UES), lower esophageal sphincter (LES), respiratory inversion point (RIP), and basal EGJ pressure. Following this, ten 5 ml wet swallows of ambient temperature water or saline (when using high resolution impedance manometry, HRIM) are performed. Finally, one multiple rapid swallow (MRS) sequence is performed (five 2 mL wet swallows administered using a 10 mL syringe 2-3 seconds apart), which can be repeated up to three times if there is a failed attempt or an abnormal contractile response. 6

Patient position is then changed to the upright position (sitting at 80 degrees or higher with legs hanging off side of bed, but not hunched or leaning over) (Figure 2B). Following the position change, a minimum of 60 seconds to allow for adaptation, a minimum of three deep inspirations to assess catheter position, and a baseline period of at least 30 seconds to enable identification of anatomic landmarks are once again performed. Next, at least five 5 ml wet swallows are performed. Finally, one rapid drink challenge (RDC) with 200 ml water, ingested as fast as possible through a straw, is performed. 7

If no conclusive evidence of a major motility disorder is identified, if results from the standard esophageal HRM protocol are not fully consistent with clinical presentation and/or findings do not explain patient symptoms, additional HRM supportive measures can be considered including solid test swallows and solid test meal to assess for EGJ obstruction, and/or post-prandial observation for rumination and/or belching disorder. 8 Pharmacologic provocation can also be performed during the initial protocol to help support a diagnosis of achalasia or opioid esophagus.(Table 2, Supplemental Figures 1-3) 9

Additionally, if equivocal results are identified and/or there is a suspicion for an EGJ outflow obstruction that does not fulfill the criteria for achalasia, a timed barium esophagram (TBE) including a barium tablet swallow and/or endoluminal functional lumen imaging probe (FLIP) should be obtained as an independent supportive test to assess EGJ obstruction. 10 11

Although the standard HRM protocol described is considered to be optimal and inclusive, clinicians can modify this protocol to adapt to available resources and time, as long as established normative values are applied and other positions and supportive measures are used appropriately. Clinicians choosing to begin the study in the upright position should perform 10 upright swallows at the outset.

Prior to the procedure, patients should fast for at least 4 hours (small amounts of clear fluid allowed) and informed consent should be obtained. The CCv4.0 Working Group recommends using a solid-state high-resolution manometry catheter with less than 2cm sensor spacing.  However, the protocol and classification can be performed with water perfused catheters if appropriate normative values are used (with the limitation of only supine swallows and maneuvers possible with water perfused manometry). Using HRIM is recommended, though not required, to optimally assess intrabolus pressure, bolus clearance and bolus flow through the EGJ. The protocol should be performed as efficiently as possible to minimize potential of pressure drift.

Diagnostic Thresholds and Definitions (Table 1)

The key HRM metrics utilized in the CCv4.0 remain assessment of deglutitive relaxation across the LES/EGJ using integrated relaxation pressure (IRP), vigor of esophageal body contraction using distal contractile integral (DCI), contractile wavefront integrity at 20 mm isobaric contour setting, and latency of deglutitive inhibition using distal latency (DL).

Integrated Relaxation Pressure:

- Threshold for median IRP in the supine position remains 15mmHg for Medtronic systems and 22mmHg for Laborie/Diversatek systems (Moderate GRADE, Strong Recommendation). 12 13 14

- Threshold for median IRP in the upright position is 12mmHg for Medtronic systems and 15mmHg for Laborie/Diversatek systems (Low GRADE, Strong Recommendation). 14, 15

Contractility in the Esophageal Body: Measured by contractile vigor and contractile pattern. 14

- Normal Contraction: DCI of 450 mmHg•s•cm to 8,000 mmHg•s•cm

- Weak Contraction: DCI between 100 mmHg•s•cm and 450 mmHg•s•cm

- Failed Peristalsis: DCI less than 100 mmHg•s•cm

- Ineffective Swallow: Includes a swallow with weak contraction or failed peristalsis. In addition, in the CCv4.0 a transition zone defect of peristalsis greater than 5cm under an isobaric contour of 20mmHg in the setting of a DCI of 450 mmHg•s•cm or greater (considered a fragmented swallow in prior Chicago Classifications) is also considered an ineffective swallow.

- Hypercontractile Swallow: DCI greater than 8,000 mmHg•s•cm, Available literature on normal values suggests that the threshold of 8,000 mmHg•s•cm is suitable across HRM systems.

Latency of Deglutitive Inhibition: DL is measured as the interval from the start of relaxation of the UES to the contractile deceleration point (CDP). The CDP is the inflection point between the proximal rapid and the distal slow phase of the esophageal contraction, located within 3 cm of the proximal aspect of the pre-swallow EGJ high-pressure zone. Physiologically this likely represents conversion from smooth muscle esophageal body contraction to the LES after-contraction.

- Premature contraction: A distal latency shorter than 4.5 seconds, in the setting of a DCI of 450 mmHg•s•cm or greater14

- If the CDP is difficult to determine, a horizontal line can be drawn 2-3 cm above the proximal aspect of the pre-swallow EGJ high-pressure zone and the DL can be determined by the duration of time from the start of the UES relaxation to the intersection at the contractile wave-front.  It is important that this horizontal line is extended to the contraction and not to the pressurization front that can be compartmentalized ahead of the peristaltic contractile wave-front.

Esophageal contractile activity must be distinguished from other causes of pressure rise in the distal esophagus such as intrabolus pressure and/or artifact. (Very Low GRADE, Strong Recommendation) 16, 17

Pressurization

The cut-off for panesophageal pressurization is set at 30 mmHg using the isobaric contour tool (Very Low GRADE, Strong Recommendation). 18 The intrabolus pressure threshold for supine wet swallows using the Medtronic system is 20mmHg (Moderate GRADE). 12, 19, 20

Response to Provocation (Table 2)

An intact response to MRS is defined as absence of esophageal body contractility (DCI < 100 mmHg•s•cm) with complete deglutitive inhibition during the repetitive swallows, and presence of post-MRS contraction augmentation (DCI post-MRS greater than single swallow mean DCI).6, 21, 22 The post-MRS contraction needs to be true peristaltic contractility and not artifact or pressurization.

An intact response to RDC is defined as absence of esophageal body contractility (DCI < 100 mmHg•s•cm) with complete deglutitive inhibition during the RDC. When assessing response to RDC, IRP >12mmHg (using Medtronic software) and panesophageal pressurization >20mmHg are criteria for outflow obstruction (Low GRADE, Conditional Recommendation).7, 23, 24 The presence of a normal contraction sequence following the RDC is a specific marker of normal contractility; however, this is not present in all healthy controls.7, 23, 24

An intact esophageal body contractile response to solid swallows requires presence of >20% pharyngeal contractions, followed by an effective esophageal contraction defined by DCI > 1000 mmHg•s•cm and without a large break (>5 cm) in the contractile front.25  Temporal association of ineffective contractions with patient symptoms (e.g. dysphagia) supports diagnosis of clinically relevant, abnormal peristaltic function.8

CHICAGO CLASSIFICATION v4.0 OF ESOPHAGEAL MOTILITY DISORDERS

The hierarchical classification scheme of the Chicago Classification is maintained in CCv4.0, whereby motility disorders are classified as disorders of EGJ function or disorders of peristalsis (Table 3, Figure 3). The classification scheme applies to patients with normal foregut anatomy, without prior surgical or invasive foregut intervention and without large hiatal hernias and/or paraesophageal hernias, as these anatomical changes can alter measurement of the CC metrics. Additionally, a careful index endoscopy is crucial prior to manometry testing, as endoscopic or radiographic evidence of obstruction precludes the use of CCv4.0. An additional update in the CCv4.0 is the emphasis that specific motility disorders should be considered clinically relevant only in the context of compatible symptoms and/or supportive testing, as detailed in this document and indicated by an asterisk in the figures.

DISORDERS OF EGJ FUNCTION (Supplemental Table 2)

ACHALASIA

Consistent with prior iterations of the Chicago Classification, CCv4.0 maintains three subtypes of achalasia: type I akin to classic achalasia, type II seen with panesophageal pressurization, and type III, or spastic, achalasia (Figure 4). A CCv4.0 update for achalasia is that an abnormal median IRP can be observed in either the supine or upright position.

Conclusive Diagnosis of Achalasia

TYPE I ACHALASIA: A conclusive diagnosis of type I achalasia is defined as absent contractility (100% failed peristalsis) and an abnormal median IRP in either the supine or upright position (Very Low GRADE, Strong Recommendation). 18, 26-29

TYPE II ACHALASIA: A conclusive diagnosis of type II achalasia is defined as absent contractility (100% failed peristalsis), an abnormal IRP in the supine or upright position and panesophageal pressurization in 20% or more swallows (Very Low GRADE, Strong Recommendation). 18, 26-29

TYPE III ACHALASIA: A conclusive diagnosis of type III achalasia is defined as an abnormal IRP in the supine or upright position and evidence of spasm (20% or more swallows with premature contraction) (Very Low GRADE, Strong Recommendation). 18, 26-29

Inconclusive Diagnosis of Achalasia & Additional Considerations (Figure 5)

- An inconclusive diagnosis of type I or II achalasia includes absent contractility with no appreciable peristalsis in the setting of a normal IRP in the supine or upright position, with or without panesophageal pressurization in 20% or more swallows (Strong Recommendation).

- An inconclusive diagnosis of achalasia includes appreciable peristalsis in the setting of an abnormal IRP in the supine or upright position, and no evidence of spasm (Strong Recommendation)

- Supportive testing with a TBE with tablet and/or FLIP should be performed in patients with an inconclusive diagnosis of achalasia in the setting of dysphagia as a presenting symptom (Very Low GRADE, Strong Recommendation). 10, 11, 30-36

- Opioids are associated with type III achalasia and patients should be studied off opiate medication if possible (Low GRADE, Conditional Recommendation). 37, 38

The following achalasia statements did not meet criteria for agreement, but are recommended considerations for HRM studies suggestive of achalasia as detailed further in the technical reviews. The pressure cut-off value for panesophageal pressurization is meant to be a guide rather than a hard threshold. The distinction between type I and II achalasia is somewhat arbitrary, and does not have important clinical implications beyond the very low levels (typically <15mmHg) seen with moderate to severe esophageal dilatation and an inability to generate emptying pressurization. Patients with panesophageal pressurization values above 70 mmHg may have embedded spasm, which may impact clinical outcome.

EGJ OUTFLOW OBSTRUCTION

A critical update in CCv4.0 is clarification and rigorous definition of EGJ outflow obstruction (EGJOO) (Figure 6). Following introduction of EGJOO as a motility disorder, nearly 10% of patients undergoing HRM were identified to have an EGJOO motility pattern.15, 39 While a proportion of EGJOO may evolve towards achalasia or represent a variant of achalasia, more than one-third of cases may be clinically irrelevant, and related to benign etiologies including mechanical effects, opiate use, and artifact as detailed in the subsequent EGJOO technical review. To avoid unnecessary treatments and optimize outcomes, there is a critical need to clarify which patients with manometric EGJOO have obstructive physiology requiring intervention. 11, 15, 19, 20, 32, 39-45 Therefore, CCv4.0 recommends the following to distinguish between clinically relevant EGJOO that may represent an underlying pathologic motor disorder responsive to treatment versus a clinically irrelevant manometric observation.

Clinically Relevant Conclusive Diagnosis of EGJOO

- A manometric diagnosis of EGJOO is always considered clinically inconclusive (Strong Recommendation).

- A clinically relevant conclusive diagnosis of EGJOO requires a manometric diagnosis of EGJOO and clinically relevant symptoms with at least one of the following supportive investigations (TBE with tablet and/or FLIP) (Moderate GRADE, Conditional Recommendation). 9-11, 15, 30

- A manometric diagnosis of EGJOO is defined as an elevated median IRP and elevated intrabolus pressure with supine wet swallows, and persistently elevated median IRP with changing position (Low GRADE, Conditional Recommendation). 9, 15, 39, 43, 44, 46-48

- Clinically relevant symptoms of EGJOO include dysphagia and/or non-cardiac chest pain (Low GRADE, Conditional Recommendation). 20, 32, 40, 42-44, 49, 50

Inconclusive Diagnosis of EGJOO & Additional Considerations

- Isolated elevated abnormal findings are inconclusive for a manometric diagnosis of EGJOO. These include isolated elevated supine IRP, isolated elevated upright IRP, or isolated elevated supine intrabolus pressure (Low GRADE, Strong Recommendation). 39, 51, 52

- A manometric diagnosis of EGJOO is supported by evidence of outflow obstruction and esophageal pressurization during the RDC and solid test meal, especially if this is temporally associated with patient symptoms (Supplemental Figure 1) (Conditional Recommendation).7, 8, 23-25, 39

- A manometric diagnosis of EGJOO is supported by evidence of abnormal EGJ function following pharmacologic provocation (Supplemental Figure 2) (Conditional Recommendation). 9

- A manometric diagnosis of EGJOO can be seen without a disorder of peristalsis, or can coexist with disorders of peristalsis such as hypercontractile esophagus and ineffective esophageal motility (Accepted Clinical Observation).

DISORDERS OF PERISTALSIS (Supplemental Table 3)

Consistent with prior iterations of Chicago Classification, absent contractility, distal esophageal spasm, hypercontractile esophagus and ineffective esophageal motility are considered disorders of peristalsis. Fragmented peristalsis is now removed as a disorder and incorporated into the overall diagnosis of IEM (Very Low GRADE, Strong Recommendation). 53, 54

An important update in CCv4.0 is the recognition that DES and hypercontractile esophagus are manometric patterns that do not always equate to a clinical disease, similar to concepts underlying EGJOO. Per CCv4.0 these disorders of peristalsis are clinically relevant only in the appropriate clinical context and when they are supported by further testing, as detailed in this section.

ABSENT CONTRACTILITY

Criteria for a diagnosis of absent contractility was not revised in CCv4.0. A conclusive diagnosis for absent contractility requires normal EGJ relaxation (normal median IRP in the supine and upright position) and 100% failed peristalsis (DCI < 100 mmHg•s•cm) (Accepted Clinical Observation) (Figure 7). In the context of absent contractility, borderline median IRP values, particularly supine median IRP of 10mmHg to 15mmHg using the Medtronic system, should prompt consideration of type I achalasia.  As discussed in the Achalasia section, supportive testing with TBE with tablet and FLIP should be considered in these cases if dysphagia is the dominant symptom (Figure 5).

DISTAL ESOPHAGEAL SPASM

DES describes a specific abnormal esophageal motor pattern characterized by spastic or premature contractions in the distal esophagus (Figure 7). As aforementioned a spastic contraction is defined as an esophageal contraction with a distal latency shorter than 4.5 seconds, in the setting of a DCI greater than 450 mmHg•s•cm. Manometric DES may have varying clinical significance, and thus, an update in CCv4.0 is the distinction between clinically relevant DES and clinically irrelevant manometric observations.

Clinically Relevant Diagnosis of Distal Esophageal Spasm

- A clinically relevant diagnosis of DES requires both clinically relevant symptoms and a conclusive manometric diagnosis of DES (Low GRADE, Conditional Recommendation). 16

- Clinically relevant symptoms for DES include dysphagia and non-cardiac chest pain (Accepted Clinical Observation).

- A conclusive manometric diagnosis of DES is defined as presence of at least 20% of premature contractions (Low GRADE, Strong Recommendation). 16

Inconclusive Diagnosis of Distal Esophageal Spasm & Additional Considerations

- The presence of at least 20% of premature contractions (DL < 4.5 seconds) but with a DCI < 450 mmHg•s•cm is inconclusive for a manometric diagnosis of DES (Low GRADE, Conditional Recommendation). 16

The CCv4.0 Working Group recognizes that the CDP might be difficult to identify. In this setting alternative methodologies need to be considered to diagnose DES (Strong Recommendation). This is further detailed in the section above on metrics and will be further addressed in the subsequent DES technical review.

HYPERCONTRACTILE ESOPHAGUS

Hypercontractile esophagus describes a distinct manometric abnormality defined by excessive peristaltic vigor, which may include excessive LES after-contraction, not associated with a mechanical obstruction (Figure 8) (Very Low GRADE, Conditional Recommendation) 55-57. Obstruction at the EGJ or the distal esophagus can induce a hypercontractile response and it is crucial that obstruction is ruled out before a diagnosis of hypercontractile esophagus is considered.  Heterogeneous motor patterns can meet manometric criteria for hypercontractile esophagus and have varying clinical significance. Therefore, similar to DES, an important update in CCv4.0 is to distinguish between clinically relevant hypercontractile esophagus versus clinically irrelevant manometric observations.

Clinically Relevant Conclusive Diagnosis of Hypercontractile Esophagus

- A clinically relevant diagnosis of hypercontractile esophagus requires both clinically relevant symptoms and a conclusive manometric diagnosis of hypercontractile esophagus (Very Low GRADE, Strong Recommendation). 49, 55, 57, 58

- Clinically relevant symptoms of hypercontractile esophagus include dysphagia and chest pain (Very Low GRADE, Strong Recommendation). 49, 55, 57, 58

- A conclusive manometric diagnosis of hypercontractile esophagus is defined as 20% or more hypercontractile supine swallows (Very Low GRADE, Conditional Recommendation). 55-57

- A diagnosis of hypercontractile esophagus can only be made when criteria for achalasia or distal esophageal spasm are not met and a mechanical obstruction has been carefully ruled out (Very Low GRADE, Strong Recommendation). 55-57

Given the heterogeneity of hypercontractile patterns, the group advocated for a cautious approach in terms of treating contractile vigor as an endpoint and advocated for conservative medical therapy before endoscopic or surgical interventions are considered. The hypercontractile esophagus working group also proposed statements which did not meet criteria for agreement, further detailed in the subsequent technical review. Overall the CCv4.0 working group recognizes the critical need for further research to better characterize physiologic phenotypes and clinical outcomes in hypercontractile esophagus. In CCv4.0, Jackhammer Esophagus is considered a subgroup of hypercontractile esophagus with repetitive prolonged contractions, especially in the post-peak phase. The Jackhammer subgroup of hypercontractile esophagus is typically associated with higher DCI values and worse symptom severity. 59-61

INEFFECTIVE ESOPHAGEAL MOTILITY

Prior iterations of Chicago Classification categorized IEM and fragmented peristalsis as minor motility disorders. In CCv4.0 fragmented peristalsis as detailed in previous iterations of the Chicago Classification is now included under the definition of IEM. Further, the diagnostic criteria for IEM have been made more stringent, commensurate with emerging data. As a result of these changes to IEM and fragmented peristalsis, CCv4.0 does not distinguish between major or minor disorders (Figure 7).

Conclusive Diagnosis of Ineffective Esophageal Motility

- A conclusive diagnosis of IEM requires more than 70% ineffective swallows (DCI 100 to 450 mmHg•s•cm or >5cm transition zone defect in peristalsis), or at least 50% failed peristalsis (DCI < 100 mmHg•s•cm) (Very Low GRADE, Strong Recommendation). 53, 62-64

Inconclusive Diagnosis of Ineffective Esophageal Motility & Additional Considerations

- The presence of 50 to 70% of ineffective swallows is inconclusive for a diagnosis of IEM. Supportive testing will strengthen confidence in IEM diagnosis in these cases (Very Low GRADE, Conditional Recommendation). 54, 65

- Supportive testing for a diagnosis of IEM could include poor bolus transit on impedance or barium esophagram (Very Low GRADE, Conditional Recommendation). 66-68

- Supportive testing for a diagnosis of IEM could include lack of contraction reserve on MRS (Very Low GRADE, Conditional Recommendation). 22

ESOPHAGO-GASTRIC JUNCTION METRICS

An advantage of modern day high-resolution esophageal pressure topography over conventional line tracing is the ability to precisely assess the EGJ barrier function at rest including the relationship between the LES, crural diaphragm (CD), and RIP, as well as the EGJ contractile integral (EGJ-CI), a measure of EGJ contractility in relation to respiration. Thus, a priority of CCv4.0 was to provide guidance to enable better characterization of the EGJ complex.

- The EGJ complex should be measured during quiet respiration in the baseline recording in a segment relatively devoid of swallowing and/or recording artifacts. This also refers to measurement of intragastric pressure, which should be measured below the CD over three complete respiratory cycles, preferably in the same segment as used to measure the EGJ-CI (Strong Recommendation).

- The RIP is the axial location at which the inspiratory change in pressure transitions from an inspiratory increase, characteristic of intra-abdominal recordings, to an inspiratory decrease, characteristic of intrathoracic recordings. (Strong Recommendation)

- The EGJ-CI should be referenced to intragastric pressure and expressed in units of mmHg•cm. (Strong Recommendation). While not met with agreement, it was suggested that an EGJ-CI (or LES-contractile integral) value of <25 mmHg•cm be considered a hypotensive EGJ. 69-76

- LES-CD separation should be scored as the distance between the center of the CD and LES signal during inspiration, unless obscured in which case the LES position should be scored at expiration (Strong Recommendation).

- The EGJ complex should be defined based on LES-CD separation and location of the RIP. (Low GRADE, Conditional Recommendation) 77-83 However, there was no consensus regarding the subtype classification scheme and two versions were deemed acceptable. 77, 82

The EGJ Metrics technical review details further recommendations regarding measurement of the EGJ complex in setting of hiatal hernia and temporal variability, as well as intragastric pressure and the role of end-expiratory LES pressure.

CONCLUSION

CCv4.0 is the state-of-the-art classification scheme of esophageal motility disorders derived from a two-year international initiative involving 52 esophageal motility experts representing professional societies from five continents. The Chicago Classification has always been a work-in-progress, subject to revision and refinement with every new iteration. True to this concept, CCv4.0 has updates that improve precision of previously characterized motility diagnoses, and deletions of infrequently encountered diagnoses or clinically irrelevant criteria. A key update in CCv4.0 is the recognition that, similar to other medical investigations, HRM patterns alone do not always equate to a conclusive diagnosis that explains patient symptoms and guides effective management (actionable pathology). Thus, CCv4.0 separates patterns that do provide a conclusive diagnosis (e.g. achalasia) from other patterns that are suggestive but inconclusive for a diagnosis, where additional clinical information and supportive testing may either confirm or refute the diagnosis in question. In particular, EGJOO, hypercontractile esophagus and DES are manometric patterns that require presence of dysphagia and/or chest pain to be considered clinically relevant. Further, a conclusive diagnosis of EGJOO requires corroboration with at least one supportive test (e.g. TBE, FLIP).

Methodologic rigor applied to the CCv4.0 process consists of the use of formal consensus methods and formal level of evidence review when applicable.  Further, CCv4.0 recommends a standardized HRM protocol to improve technical consistency and diagnostic accuracy, so that future iterations of the Chicago Classification can rely on further research from a comprehensive, uniformly collected data. Finally, CCv4.0 represents motility perspectives from a diverse working group in terms of geography, age, gender, practice type, years in practice, and research contributions to the field.

CCv4.0 also highlights areas ripe for future investigation and clarity, as further summarized in the specific technical reviews. Future iterations of Chicago Classification will need to delineate the role of impedance topography for intrabolus pressure and bolus flow. Additional outcomes studies are needed to better understand the reliability of solid test swallows and meals in identifying clinically relevant abnormal EGJ and peristaltic function. With refinement of the diagnostic criteria for EGJOO in CCv4.0, it will be important to understand the natural history and treatment outcome of EGJOO, with and without supportive testing. Understanding mechanisms of spastic esophageal disorders remains of great interest, as well as exploring overlaps with opioid induced esophageal dysfunction.38 Understanding the spectrum of hypercontractile disorders will require further work to explore whether jackhammer esophagus represents a unique subtype with clinical significance.  Future iterations of Chicago Classification may propose manometric criteria for therapy selection, such as role of per-oral endoscopic myotomy for spastic disorders, and risk stratification and tailoring of fundoplication to prevent post-fundoplication dysphagia.

FIGURE AND TABLE LEGEND

TABLES

Table 1. HRM Metrics and Thresholds

Table 2. Supportive Manometric Measures which may Increase Confidence for a Disorder

Table 3. Classification and Definition of Manometric Disorders

FIGURES

Figure 1. Standard high-resolution esophageal manometry protocol per CCv4.0

Figure 2. High-resolution manometry images depicted the standard protocol. 2A) The supine position includes a 60 second adaptation period, 3 deep breaths, 30 second baseline period, 10 five ml wet swallows and at least one multiple rapid swallow. 2B) Position is changed to the upright position followed by a 60 second adaptation, 3 deep breaths, 30 second baseline period, 5 five ml wet swallows and a rapid drink challenge.

Figure 3. The Chicago Classification 4.0 Hierarchical Classification Scheme.  This flow diagram highlights how the CC 4.0 diagnosis will be defined by going through the various phases of the protocol.  The current protocol allows for some flexibility if the diagnosis is conclusive with 10 swallows in the supine and upright position and allows for a sequenced progression of the protocol to help confirm or rule out the diagnosis.  ‡ Pharmacologic provocation can be instituted here during the protocol if available to assess for the presence of achalasia or mechanical obstruction.  ¥ A solid test meal or post-prandial challenge with a symptom inducing meal can be added if no evidence of a disorder of peristalsis or EGJ obstruction in patients with regurgitation, belching or a high probability of a missed EGJOO.

Figure 4. Achalasia Subtypes. Type I Achalasia: IRP is elevated with failed peristalsis (DCI < 100 mmHg-s-cm), and without panesophageal pressurization. Type II Achalasia: IRP is elevated with failed peristalsis and panesophageal pressurization. Type III Achalasia: IRP is elevated with a normal DCI, and a reduced distal latency.

Figure 5. Inconclusive Diagnosis for Achalasia or Absent Contractility Requires Supportive Testing. Findings are inconclusive for type I achalasia or absent contractility as there is 100% failed peristalsis but the median IRP is at the upper limit of normal with 5ml wet swallows. With the rapid drink challenge there is absence of deglutitive inhibition across the LES. Supportive testing is required in the setting of inconclusive findings with timed barium esophagram and/or FLIP. Here the timed barium esophagram demonstrates a dilated distal esophagus with barium retention. On FLIP the EGJ distensibility index is reduced, maximal EGJ diameter is reduced and there is absent contractile response to distension.

Figure 6. EGJOO sub-types: EGJOO with hypercontractile esophagus: IRP is elevated with intrabolus pressurization and hypercontractile swallow. EGJOO without disorder of peristalsis. IRP is elevated with normal contractile vigor. Manometric EGJOO related to artifactual rise in IRP: IRP is elevated in the absence of intrabolus pressurization, and is likely associated with artifact.

Figure 7. Disorders of Peristalsis with Reduced Contractile Vigor or Contiguity of Peristalsis.  These include absent contractility or ineffective esophageal motility (either related to reduced contractile vigor or fragmented peristalsis). In this example of Absent Contractility there is failed peristalsis with a normal IRP. In the first example of IEM the DCI is reduced with a normal IRP. In the second example of IEM the DCI is normal with a fragmentation in peristalsis of > 5cm in the setting of a normal IRP. During the multiple rapid swallow there is absence of contractile activity and there is deglutitive inhibition of LES followed by DCI which is greater than the single swallow DCI, signifying an intact contractile augmentation.

Figure 8. Disorders of Peristalsis with Esophageal Spasticity or Hypercontractility. These include distal esophageal spasm or hypercontractile esophagus. In this example of Distal Esophageal Spasm the DCI is normal with a reduced distal latency and normal IRP. Hypercontractile esophagus includes sub-groups: Hypercontractile (based on elevated DCI), Hypercontractile with Jackhammer Esophagus pattern, and Hypercontractile with LES after-contraction.

SUPPLEMENTAL MATERIALS

Supplemental Table 1 Working Group Baseline Characteristics and Responses to Initial Survey

Supplemental Table 2. Disorders of EGJ Function. Definitions of a conclusive diagnosis, inconclusive diagnosis and supportive testing.

Supplemental Table 3. Disorders of Esophageal Peristalsis. Definitions of a conclusive diagnosis, inconclusive diagnosis and supportive testing.

Supplemental Table 4. Recommendations of the Chicago Classification v4.0

Supplemental Figure 1. Supportive Role of Solid Test Swallows and Solid Test Meal. In 5ml wet swallow the DCI and IRP are normal. With solid test swallows the IRP is elevated and symptoms of dysphagia are elicited, unmasking an obstruction and supporting a diagnosis of EGJ outflow obstruction in an otherwise normal study.

Supplemental Figure 2. Supportive Role of Pharmacologic Provocation

Supplemental Figure 3. Supportive Role of Post-Prandial Monitoring. In Post-prandial monitoring there is a transient LES relaxation with reflux with symptoms of reflux/belching, followed by a rumination episode with symptom of regurgitation.

REFERENCES

1.	Kahrilas PJ, Bredenoord AJ, Fox M, et al. The Chicago Classification of esophageal motility disorders, v3.0. Neurogastroenterol Motil 2015;27:160-74.

2.	Pandolfino JE, Fox MR, Bredenoord AJ, et al. High-resolution manometry in clinical practice: utilizing pressure topography to classify oesophageal motility abnormalities. Neurogastroenterol Motil 2009;21:796-806.

3.	Bredenoord AJ, Fox M, Kahrilas PJ, et al. Chicago classification criteria of esophageal motility disorders defined in high resolution esophageal pressure topography. Neurogastroenterol Motil 2012;24 Suppl 1:57-65.

4.	Yadlapati R, Gawron AJ, Keswani RN, et al. Identification of Quality Measures for Performance of and Interpretation of Data From Esophageal Manometry. Clin Gastroenterol Hepatol 2016;14:526-534 e1.

5.	Yadlapati R, Vaezi MF, Vela MF, et al. Management options for patients with GERD and persistent symptoms on proton pump inhibitors: recommendations from an expert panel. Am J Gastroenterol 2018;113:980-986.

6.	Shaker A, Stoikes N, Drapekin J, et al. Multiple rapid swallow responses during esophageal high-resolution manometry reflect esophageal body peristaltic reserve. Am J Gastroenterol 2013;108:1706-12.

7.	Woodland P, Gabieta-Sonmez S, Arguero J, et al. 200 mL Rapid Drink Challenge During High-resolution Manometry Best Predicts Objective Esophagogastric Junction Obstruction and Correlates With Symptom Severity. J Neurogastroenterol Motil 2018;24:410-414.

8.	Ang D, Misselwitz B, Hollenstein M, et al. Diagnostic yield of high-resolution manometry with a solid test meal for clinically relevant, symptomatic oesophageal motility disorders: serial diagnostic study. Lancet Gastroenterol Hepatol 2017;2:654-661.

9.	Babaei A, Shad S, Szabo A, et al. Pharmacologic interrogation of patients with esophagogastric junction outflow obstruction using amyl nitrite. Neurogastroenterol Motil 2019;31:e13668.

10.	Triggs JR, Carlson DA, Beveridge C, et al. Functional Luminal Imaging Probe Panometry Identifies Achalasia-Type Esophagogastric Junction Outflow Obstruction. Clin Gastroenterol Hepatol 2020;18:2209-2217.

11.	Clayton SB, Patel R, Richter JE. Functional and Anatomic Esophagogastic Junction Outflow Obstruction: Manometry, Timed Barium Esophagram Findings, and Treatment Outcomes. Clin Gastroenterol Hepatol 2016;14:907-911.

12.	Herregods TV, Roman S, Kahrilas PJ, et al. Normative values in esophageal high-resolution manometry. Neurogastroenterol Motil 2015;27:175-87.

13.	Kuribayashi S, Iwakiri K, Kawada A, et al. Variant parameter values-as defined by the Chicago Criteria-produced by ManoScan and a new system with Unisensor catheter. Neurogastroenterol Motil 2015;27:188-94.

14.	Sifrim D, Roman S, Savarino E, et al. Normal values and regional differences in oesophageal impedance-pH metrics: a consensus analysis of impedance-pH studies from around the world. Gut 2020.

15.	Triggs JR, Carlson DA, Beveridge C, et al. Upright Integrated Relaxation Pressure Facilitates Characterization of Esophagogastric Junction Outflow Obstruction. Clin Gastroenterol Hepatol 2019;17:2218-2226 e2.

16.	Pandolfino JE, Roman S, Carlson D, et al. Distal esophageal spasm in high-resolution esophageal pressure topography: defining clinical phenotypes. Gastroenterology 2011;141:469-75.

17.	Roman S, Kahrilas PJ, Kia L, et al. Effects of large hiatal hernias on esophageal peristalsis. Arch Surg 2012;147:352-7.

18.	Pandolfino JE, Kwiatek MA, Nealis T, et al. Achalasia: a new clinically relevant classification by high-resolution manometry. Gastroenterology 2008;135:1526-33.

19.	Biasutto D, Mion F, Garros A, et al. Rapid drink challenge test during esophageal high resolution manometry in patients with esophago-gastric junction outflow obstruction. Neurogastroenterol Motil 2018;30:e13293.

20.	Song BG, Min YW, Lee H, et al. Clinicomanometric factors associated with clinically relevant esophagogastric junction outflow obstruction from the Sandhill high-resolution manometry system. Neurogastroenterol Motil 2018;30.

21.	Elvevi A, Mauro A, Pugliese D, et al. Usefulness of low- and high-volume multiple rapid swallowing during high-resolution manometry. Dig Liver Dis 2015;47:103-7.

22.	Martinucci I, Savarino EV, Pandolfino JE, et al. Vigor of peristalsis during multiple rapid swallows is inversely correlated with acid exposure time in patients with NERD. Neurogastroenterol Motil 2016;28:243-50.

23.	Ang D, Hollenstein M, Misselwitz B, et al. Rapid Drink Challenge in high-resolution manometry: an adjunctive test for detection of esophageal motility disorders. Neurogastroenterol Motil 2017;29.

24.	Marin I, Serra J. Patterns of esophageal pressure responses to a rapid drink challenge test in patients with esophageal motility disorders. Neurogastroenterol Motil 2016;28:543-53.

25.	Hollenstein M, Thwaites DT, Buetikofer S, et al. Pharyngeal swallowing and oesophageal motility during a solid meal test: a prospective study in healthy volunteers and patients with major motility disorders. The Lancet Gastroenterology & Hepatology 2017;2:644-653.

26.	Andolfi C, Fisichella PM. Meta-analysis of clinical outcome after treatment for achalasia based on manometric subtypes. Br J Surg 2019;106:332-341.

27.	Lee JY, Kim N, Kim SE, et al. Clinical characteristics and treatment outcomes of 3 subtypes of achalasia according to the chicago classification in a tertiary institute in Korea. J Neurogastroenterol Motil 2013;19:485-94.

28.	Min M, Peng LH, Yang YS, et al. Characteristics of achalasia subtypes in untreated Chinese patients: a high-resolution manometry study. J Dig Dis 2012;13:504-9.

29.	Rohof WO, Salvador R, Annese V, et al. Outcomes of treatment for achalasia depend on manometric subtype. Gastroenterology 2013;144:718-25; quiz e13-4.

30.	Blonski W, Kumar A, Feldman J, et al. Timed Barium Swallow: Diagnostic Role and Predictive Value in Untreated Achalasia, Esophagogastric Junction Outflow Obstruction, and Non-Achalasia Dysphagia. Am J Gastroenterol 2018;113:196-203.

31.	Clayton SB, Shin CM, Ewing A, et al. Pneumatic dilation improves esophageal emptying and symptoms in patients with idiopathic esophago-gastric junction outflow obstruction. Neurogastroenterol Motil 2019;31:e13522.

32.	Okeke FC, Raja S, Lynch KL, et al. What is the clinical significance of esophagogastric junction outflow obstruction? evaluation of 60 patients at a tertiary referral center. Neurogastroenterol Motil 2017;29.

33.	Carlson DA, Kahrilas PJ, Lin Z, et al. Evaluation of Esophageal Motility Utilizing the Functional Lumen Imaging Probe. Am J Gastroenterol 2016;111:1726-1735.

34.	Kim E, Yoo IK, Yon DK, et al. Characteristics of a Subset of Achalasia With Normal Integrated Relaxation Pressure. J Neurogastroenterol Motil 2020;26:274-280.

35.	Ponds FA, Bredenoord AJ, Kessing BF, et al. Esophagogastric junction distensibility identifies achalasia subgroup with manometrically normal esophagogastric junction relaxation. Neurogastroenterol Motil 2017;29.

36.	Sanagapalli S, Roman S, Hastier A, et al. Achalasia diagnosed despite normal integrated relaxation pressure responds favorably to therapy. Neurogastroenterol Motil 2019;31:e13586.

37.	Babaei A, Szabo A, Shad S, et al. Chronic daily opioid exposure is associated with dysphagia, esophageal outflow obstruction, and disordered peristalsis. Neurogastroenterol Motil 2019;31:e13601.

38.	Ratuapli SK, Crowell MD, DiBaise JK, et al. Opioid-Induced Esophageal Dysfunction (OIED) in Patients on Chronic Opioids. Am J Gastroenterol 2015;110:979-84.

39.	Misselwitz B, Hollenstein M, Butikofer S, et al. Prospective serial diagnostic study: the effects of position and provocative tests on the diagnosis of oesophageal motility disorders by high-resolution manometry. Aliment Pharmacol Ther 2020;51:706-718.

40.	Beveridge CA, Falk GW, Ahuja NK, et al. Low Yield of Cross-Sectional Imaging in Patients With Esophagogastric Junction Outflow Obstruction. Clin Gastroenterol Hepatol 2020;18:1643-1644.

41.	DeLay K, Austin GL, Menard-Katcher P. Anatomic abnormalities are common potential explanations of manometric esophagogastric junction outflow obstruction. Neurogastroenterol Motil 2016;28:1166-71.

42.	Liu A, Woo M, Nasser Y, et al. Esophagogastric junction outflow obstruction on manometry: Outcomes and lack of benefit from CT and EUS. Neurogastroenterol Motil 2019;31:e13712.

43.	Perez-Fernandez MT, Santander C, Marinero A, et al. Characterization and follow-up of esophagogastric junction outflow obstruction detected by high resolution manometry. Neurogastroenterol Motil 2016;28:116-26.

44.	van Hoeij FB, Smout AJ, Bredenoord AJ. Characterization of idiopathic esophagogastric junction outflow obstruction. Neurogastroenterol Motil 2015;27:1310-6.

45.	Wang YT, Tai LF, Yazaki E, et al. Investigation of Dysphagia After Antireflux Surgery by High-resolution Manometry: Impact of Multiple Water Swallows and a Solid Test Meal on Diagnosis, Management, and Clinical Outcome. Clin Gastroenterol Hepatol 2015;13:1575-83.

46.	Hoscheit M, Gabbard S. Elevated Intrabolus Pressure Predicts Abnormal Timed Barium Esophagram in Esophagogastric Junction Outflow Obstruction. J Neurogastroenterol Motil 2019;25:521-524.

47.	Quader F, Reddy C, Patel A, et al. Elevated intrabolus pressure identifies obstructive processes when integrated relaxation pressure is normal on esophageal high-resolution manometry. Am J Physiol Gastrointest Liver Physiol 2017;313:G73-G79.

48.	Singendonk MJ, Lin Z, Scheerens C, et al. High-resolution impedance manometry parameters in the evaluation of esophageal function of non-obstructive dysphagia patients. Neurogastroenterol Motil 2019;31:e13505.

49.	Schupack D, Katzka DA, Geno DM, et al. The clinical significance of esophagogastric junction outflow obstruction and hypercontractile esophagus in high resolution esophageal manometry. Neurogastroenterol Motil 2017;29:1-9.

50.	Song BG, Min YW, Lee H, et al. Combined Multichannel Intraluminal Impedance and High-resolution Manometry Improves Detection of Clinically Relevant Esophagogastric Junction Outflow Obstruction. J Neurogastroenterol Motil 2019;25:75-81.

51.	Babaei A, Lin EC, Szabo A, et al. Determinants of pressure drift in Manoscan() esophageal high-resolution manometry system. Neurogastroenterol Motil 2015;27:277-84.

52.	Babaei A, Szabo A, Yorio SD, et al. Pressure exposure and catheter impingement affect the recorded pressure in the Manoscan 360 system. Neurogastroenterol Motil 2018.

53.	Zerbib F, Marin I, Cisternas D, et al. Ineffective esophageal motility and bolus clearance. A study with combined high-resolution manometry and impedance in asymptomatic controls and patients. Neurogastroenterol Motil 2020;32:e13876.

54.	Rogers BD, Rengarajan A, Mauro A, et al. Fragmented and failed swallows on esophageal high-resolution manometry associate with abnormal reflux burden better than weak swallows. Neurogastroenterol Motil 2020;32:e13736.

55.	Herregods TV, Smout AJ, Ooi JL, et al. Jackhammer esophagus: Observations on a European cohort. Neurogastroenterol Motil 2017;29.

56.	Kahn A, Al-Qaisi MT, Obeid RA, et al. Clinical features and long-term outcomes of lower esophageal sphincter-dependent and lower esophageal sphincter-independent jackhammer esophagus. Neurogastroenterol Motil 2019;31:e13507.

57.	Roman S, Pandolfino JE, Chen J, et al. Phenotypes and clinical context of hypercontractility in high-resolution esophageal pressure topography (EPT). Am J Gastroenterol 2012;107:37-45.

58.	Al-Qaisi MT, Siddiki HA, Crowell MD, et al. The clinical significance of hypercontractile peristalsis: comparison of high-resolution manometric features, demographics, symptom presentation, and response to therapy in patients with Jackhammer esophagus versus Nutcracker esophagus. Dis Esophagus 2017;30:1-7.

59.	Philonenko S, Roman S, Zerbib F, et al. Jackhammer esophagus: Clinical presentation, manometric diagnosis, and therapeutic results-Results from a multicenter French cohort. Neurogastroenterol Motil 2020:e13918.

60.	Xiao Y, Carlson DA, Lin Z, et al. Jackhammer esophagus: Assessing the balance between prepeak and postpeak contractile integral. Neurogastroenterol Motil 2018;30:e13262.

61.	Xiao Y, Carlson DA, Lin Z, et al. Chaotic peak propagation in patients with Jackhammer esophagus. Neurogastroenterol Motil 2020;32:e13725.

62.	Bulsiewicz WJ, Kahrilas PJ, Kwiatek MA, et al. Esophageal pressure topography criteria indicative of incomplete bolus clearance: a study using high-resolution impedance manometry. Am J Gastroenterol 2009;104:2721-8.

63.	Jain A, Baker JR, Chen JW. In ineffective esophageal motility, failed swallows are more functionally relevant than weak swallows. Neurogastroenterol Motil 2018;30:e13297.

64.	Roman S, Lin Z, Kwiatek MA, et al. Weak peristalsis in esophageal pressure topography: classification and association with Dysphagia. Am J Gastroenterol 2011;106:349-56.

65.	Rengarajan A, Bolkhir A, Gor P, et al. Esophagogastric junction and esophageal body contraction metrics on high-resolution manometry predict esophageal acid burden. Neurogastroenterol Motil 2018;30:e13267.

66.	Chugh P, Collazo T, Dworkin B, et al. Ineffective Esophageal Motility Is Associated with Impaired Bolus Clearance but Does Not Correlate with Severity of Dysphagia. Dig Dis Sci 2019;64:811-814.

67.	Lin Z, Nicodeme F, Lin CY, et al. Parameters for quantifying bolus retention with high-resolution impedance manometry. Neurogastroenterol Motil 2014;26:929-36.

68.	Wu JF, Tsai IJ, Tong TW, et al. Pressure-impedance analysis: Assist the diagnosis and classification of ineffective esophageal motility disorder. J Gastroenterol Hepatol 2020;35:1317-1324.

69.	Gor P, Li Y, Munigala S, et al. Interrogation of esophagogastric junction barrier function using the esophagogastric junction contractile integral: an observational cohort study. Dis Esophagus 2016;29:820-828.

70.	Ham H, Cho YK, Lee HH, et al. Esophagogastric junction contractile integral and morphology: Two high-resolution manometry metrics of the anti-reflux barrier. J Gastroenterol Hepatol 2017;32:1443-1449.

71.	Jasper D, Freitas-Queiroz N, Hollenstein M, et al. Prolonged measurement improves the assessment of the barrier function of the esophago-gastric junction by high-resolution manometry. Neurogastroenterol Motil 2017;29.

72.	Nicodeme F, Pipa-Muniz M, Khanna K, et al. Quantifying esophagogastric junction contractility with a novel HRM topographic metric, the EGJ-Contractile Integral: normative values and preliminary evaluation in PPI non-responders. Neurogastroenterol Motil 2014;26:353-60.

73.	Rengarajan A, Gyawali CP. High-resolution Manometry can Characterize Esophagogastric Junction Morphology and Predict Esophageal Reflux Burden. J Clin Gastroenterol 2020;54:22-27.

74.	Tolone S, De Bortoli N, Marabotto E, et al. Esophagogastric junction contractility for clinical assessment in patients with GERD: a real added value? Neurogastroenterol Motil 2015;27:1423-31.

75.	Wang D, Patel A, Mello M, et al. Esophagogastric junction contractile integral (EGJ-CI) quantifies changes in EGJ barrier function with surgical intervention. Neurogastroenterol Motil 2016;28:639-46.

76.	Xie C, Wang J, Li Y, et al. Esophagogastric Junction Contractility Integral Reflect the Anti-reflux Barrier Dysfunction in Patients with Gastroesophageal Reflux Disease. J Neurogastroenterol Motil 2017;23:27-33.

77.	Akimoto S, Singhal S, Masuda T, et al. Classification for esophagogastric junction (EGJ) complex based on physiology. Dis Esophagus 2017;30:1-6.

78.	Akimoto S, Singhal S, Masuda T, et al. Esophagogastric Junction Morphology and Distal Esophageal Acid Exposure. Dig Dis Sci 2016;61:3537-3544.

79.	Mittal RK, Zifan A, Kumar D, et al. Functional morphology of the lower esophageal sphincter and crural diaphragm determined by three-dimensional high-resolution esophago-gastric junction pressure profile and CT imaging. Am J Physiol Gastrointest Liver Physiol 2017;313:G212-G219.

80.	Nicodeme F, Lin Z, Pandolfino JE, et al. Esophagogastric Junction pressure morphology: comparison between a station pull-through and real-time 3D-HRM representation. Neurogastroenterol Motil 2013;25:e591-8.

81.	Niebisch S, Wilshire CL, Peters JH. Systematic analysis of esophageal pressure topography in high-resolution manometry of 68 normal volunteers. Dis Esophagus 2013;26:651-60.

82.	Pandolfino JE, Kim H, Ghosh SK, et al. High-resolution manometry of the EGJ: an analysis of crural diaphragm function in GERD. Am J Gastroenterol 2007;102:1056-63.

83.	Tolone S, de Cassan C, de Bortoli N, et al. Esophagogastric junction morphology is associated with a positive impedance-pH monitoring in patients with GERD. Neurogastroenterol Motil 2015;27:1175-82.